Payson Pines Firewise Home Checklist

Yes        No

□    □   1.  House number visable from the street, and at night.

Ex. Curbside, on house, reflective, or lighted.

□    □   2. Has dead vegetation within the recommended space been removed?

Dead trees, limbs, pine needles, cleaned rain gutters.

Firewood and other combustibles at least 30 Ft. away.

No flammables under a deck.

□    □   3.  Have Ladder fuels been removed?

Vertical separation between vegetation layers.

No limbs hanging over the roof.

Pruned lower limbs and brush of vegetation.

□    □   4.  The area surrounding structures “ lean and clean” out to 30 Ft.

Lean- small amounts of flammable veg. and plants are healthy.

Limit, remove vegetation between homes of close proximity.

Clean- Accumulation of dead vegetation or other flammable debris.

□    □   5.  Vegetation and fuels regularly maintained?

Remove- Rake up needles, leaves and litter, but leave a layer of duff.

Reduce- Prune or trim trees as needed.

Relocate- Firewood and other combustible material

□    □   6.  Roofing material fire resistant?

Vents screened

Everyone in Payson Pines meets this minimum level.

□    □         Extra Credit, Metal roofing, superior, longer life.

Refer to Handout, “ Living with Wildfire”  for more information

No cost assessment provided on request.

FireWise Coordinator, Robert Rick, 928-978-0277